# Partitioning your design is the process of specifying the type of simulator that should be used for each cell in your design hierarchy

# This is done through the Cadence Hierarchy Editor, which makes a new cellview called the config view

# Go to the Library Manager and create a new cellview for the cell you want to simulate using File®New®Cellview

* Specify config as the View Name and Hierarchy-Editor as the Tool and hit OK

# This brings up two dialog boxes, the New Hierarchy dialog and the Hierarchy Editor dialog.

* In the New Hierarchy dialog, choose a Template from the Built-In list that best describes the simulator that you will be using.  This automatically fills in a lot of default information in the View List and Stop List that is basically correct for that particular simulator.# At this point the schematic view and config view are ready for simulation.  Simulations are setup through the standard Analog Environment dialog box as before.

# Open Analog Environment from the schematic view by choosing Tools®Analog Environment

# Make sure that the Design parameters are correct, particularly that the View is config

# Setup the simulator that is to be used by choosing Setup®Simulator/Directory/Host...

* Specify a mixed-mode Simulator such as spectreSverilog or hspiceSverilog (the exact simulator depends on the view types and/or template that you specified in the Hierarchy Editor)